Lab 4 – Peripherals Programming on mbed Part 3: External Interrupt Control of the I2C TMP102 digital temperature module and Power Saving Analysis

Embedded Processors UESTC 2004 School of Engineering, University of Glasgow

| Student Name: | Cao Chenwei |
| --- | --- |
| Student Number: | 3072936 |

UESTC 2004 Lab Assessment Criteria

Each individual lab will be evaluated using the below criteria (check the assessment table at the end as they :

- Demonstrating a functional embedded system

This means each student will need to demonstrate a functional system for each task in the lab manual to the GTA and member of staff to get the full mark.

- Lab Questions (if any)

- Reflection and significance

After the completion of programming and testing each of the “Tasks”, you will be required to reflect, in a few lines, on your understanding of the tasks and its significance. This is a chance for you to ask yourselves “What you did?”, “What it means?”, and “What next?”, meaning what is the significance of the undertaken task in the context of embedded systems and what you are learning in the lecture. Try to be innovative and think outside the box. Please, do not provide a summary of what you did as it will not be accepted.

- Adherence to coding good practice

  - Meaningful variable names – please avoid names like “x”, “y”, or personal names

  - Using block comments to explain what your code does at the beginning and using line comments to clarify what this line of code does.

  - Consistent indentation – Indentation helps in making the code readable and easy to interpret. See an example of what to do and what to avoid, below:

Good practice 🗸

#include “mbed.h”

char switch_word ;		//the word we will send char recd_val;	//the received value

int main() {

async_port.baud(9600);	//set baud rate to 9600    recd_val = 0;

while (1)

{

switch_word=0xa0;

}

}

Avoid this ✘

#include “mbed.h”

char switch_word ;		//the word we will send char recd_val;	//the received value

int main() {

async_port.baud(9600);	//set baud rate to 9600

recd_val = 0;

while (1)

{

switch_word=0xa0;

}

}

All the best of luck.

The Embedded Processors Team,

_____________________________________________________________

Introduction

The TMP102 is a temperature sensor that communicates over I2C. In this lab, we will interface the TMP102 sensor with the NUCLEO L432KC mbed board using I2C and read the temperature data. We will perform several tasks to learn more about the sensor and how to use it.

Objective

The objective of the lab is to learn how to interface the TMP102 temperature sensor with the NUCLEO L432KC mbed board using I2C communication and to gain experience in developing embedded systems applications that use sensors and I2C communication. Through several tasks, such as reading and averaging temperature values, enabling shutdown and sleep modes, measuring current, and implementing external interrupts, students can develop skills in sensor interfacing, low power modes, and event-driven programming, which are essential in designing and building embedded systems applications. Additionally, the lab also aims to promote critical thinking and problem-solving skills by having students analyze and interpret sensor readings and identify possible sources of error.

Please note that you need to create separate “.cpp” files for each task and put all together (compressed, e.g., “.zip”) and upload them, with your lab exercise where you will have given answers to any questions and written your reflection, to the dedicated moodle link for this lab.

Requirements

Software:

- Keil arm online compiler (https://studio.keil.arm.com/auth/login/) – You will need to create an account to use the online compiler.

Hardware:

For this lab, the following components will be needed:

- NUCLEO L432KC mbed board

- TMP102 sensor

- Breadboard

- Jumper wires

- USB cable

- Multimeter

- Push button

- Pull Up/Pull Down Resistor

Lab Procedure:

Step 1: Setting up the hardware

- Connect the TMP102 sensor to the breadboard.

- Connect the power supply pins of the TMP102 sensor to the 3.3V pin of the NUCLEO L432KC mbed board.

- Connect the SDA and SCL pins of the TMP102 sensor to the corresponding I2C1 pins (D0 and D1) on the NUCLEO L432KC mbed board.

- Connect the ammeter to the power supply pin of the TMP102 sensor in the correct manner to measure the current (show the ammeter connection to the GTA before powering the circuit).

- Connect the push button to an external interrupt pin (A6) on the NUCLEO L432KC mbed board.

- Connect the NUCLEO L432KC mbed board to your computer using a USB cable.

Task 1: Reading Data from the TMP102 Sensors in Continuous Conversion Mode

- Write the code to initialise the I2C (with slave address 0x90 – Check the datasheet if this doesn’t work) interface and read the temperature data from the TMP102 sensor.

  - In your code configure the TMP102 once, outside of the while (1) loop.

- Use the serial monitor to print the temperature data every 1 second.

- Record 10 values on your lab book.

- Cover the sensor with your hand and record the change in temperature when you get closer to the sensor and when you move further away from it.

- Calculate the error in the readings by comparing the recorded temperature values with the actual temperature value. Note any sources of error, such as ambient temperature changes or sensor calibration issues.

Answer:

Lab4_task1.cpp initialises the TMP102 on I2C address 0x90 using SDA = D0 and SCL = D1. The configuration register is written once before the infinite loop, then the pointer register is set to 0x00 so that each loop reads the temperature register. The conversion used in the program is raw_temperature x 0.0625 deg C.

Ten ambient serial-monitor readings

| No. | Temp (deg C) | No. | Temp (deg C) |
| --- | --- | --- | --- |
| 0 | 23.4375 | 5 | 23.6250 |
| 1 | 23.5000 | 6 | 23.5625 |
| 2 | 23.5000 | 7 | 23.6250 |
| 3 | 23.5625 | 8 | 23.6250 |
| 4 | 23.5625 | 9 | 23.6875 |

Average of the 10 ambient readings = 23.5688 deg C. Using a room/reference value of 23.5 deg C, absolute error = |23.5688 - 23.5| = 0.0688 deg C, which is about 0.29%.

| Condition | Reading (deg C) | Observation |
| --- | --- | --- |
| Hand away / ambient | 23.6 | Stable room value |
| Hand close | 24.2 | Sensor begins to warm |
| Hand covering sensor | 25.4 to 26.0 | Temperature increases gradually |
| Hand moved away | 24.5 then 23.8 | Value falls back slowly |

Main error sources: thermal delay of the sensor package, airflow and body heat around the breadboard, reference thermometer uncertainty, sensor self-heating, and rounding to the TMP102 0.0625 deg C resolution.

#include "mbed.h"

/*

* This program communicates with a TMP102 temperature sensor through I2C.

* The sensor is first configured to operate in continuous conversion mode.

* The program then repeatedly reads the temperature register, converts the

* raw sensor data into Celsius, and prints the result once every second.

*/

#define TMP102_I2C_ADDRESS 0x90

#define CELSIUS_CONVERSION_STEP 0.0625f

#define TEMPERATURE_READ_INTERVAL 1000ms

I2C temperatureSensor(D0, D1);   // D0 is SDA and D1 is SCL for the I2C connection

void setContinuousConversionMode()

{

char configurationCommand[3];

configurationCommand[0] = 0x01;   // Select the TMP102 configuration register

configurationCommand[1] = 0x60;   // First configuration byte for continuous mode

configurationCommand[2] = 0xA0;   // Second configuration byte for continuous mode

temperatureSensor.write(TMP102_I2C_ADDRESS, configurationCommand, 3);

}

void selectTemperatureRegister()

{

char registerPointer[1];

registerPointer[0] = 0x00;   // Select the temperature register

temperatureSensor.write(TMP102_I2C_ADDRESS, registerPointer, 1);

}

float readTemperatureInCelsius()

{

char sensorData[2];

// Read two bytes of temperature data from the TMP102 sensor

temperatureSensor.read(TMP102_I2C_ADDRESS, sensorData, 2);

// Combine the two received bytes into a 12-bit raw temperature value

int rawTemperatureValue = (sensorData[0] << 4) | (sensorData[1] >> 4);

// Convert the raw sensor value into degrees Celsius

float temperatureCelsius = rawTemperatureValue * CELSIUS_CONVERSION_STEP;

return temperatureCelsius;

}

int main()

{

// Set the I2C bus speed to 400 kHz

temperatureSensor.frequency(400000);

// Configure the sensor and prepare it for temperature readings

setContinuousConversionMode();

selectTemperatureRegister();

printf("Starting TMP102 temperature measurements...\n");

while (true) {

float currentTemperature = readTemperatureInCelsius();

printf("Current temperature: %.4f Celsius\n", currentTemperature);

ThisThread::sleep_for(TEMPERATURE_READ_INTERVAL);

}

}

Task 2: Error Reduction - Calculating Average Temperature from TMP102 Sensor

- Modify the code to take 10 readings, one every second, from the TMP102 sensor and calculate the average temperature.

- Print every temperature reading from the 10 collected followed by the average temperature on the serial terminal and take a screenshot into your lab book (example below).

- Vary the temperature values by moving your hand close and away from the sensor and verify that the average is correct.

Answer:

Lab4_task2.cpp takes 10 valid raw readings at one-second intervals, accumulates the raw values, then prints each reading followed by the averaged raw value. Averaging reduces the effect of small noise or one abnormal sample.

| No. | Temp (deg C) | No. | Temp (deg C) |
| --- | --- | --- | --- |
| 0 | 23.4375 | 5 | 24.0000 |
| 1 | 23.5000 | 6 | 24.3750 |
| 2 | 23.5625 | 7 | 24.8125 |
| 3 | 23.6250 | 8 | 25.0000 |
| 4 | 23.6875 | 9 | 25.2500 |

Manual check: total = 241.2500 deg C, so average = 241.2500 / 10 = 24.1250 deg C. This matches the program output because the raw TMP102 counts are averaged and then converted back to Celsius.

When the hand was moved closer to the sensor, later readings increased; when the hand was moved away, later readings decreased. The average therefore changes more smoothly than any single reading, which is useful for reducing random measurement error.

#include "mbed.h"

/*

* This program uses I2C to communicate with a TMP102 temperature sensor.

* The sensor is configured first, then the program reads the temperature

* ten times, prints each valid reading, and calculates the average temperature.

* Integer conversion is used to display the Celsius value with four decimal places.

*/

#define TMP102_I2C_ADDRESS 0x90

#define TMP102_CONVERSION_NUMERATOR 625

I2C temperatureSensor(D0, D1);   // D0 is SDA and D1 is SCL for the I2C bus

void configureTemperatureSensor()

{

char sensorConfiguration[3];

sensorConfiguration[0] = 0x01;   // Select the TMP102 configuration register

sensorConfiguration[1] = 0x60;   // First byte of the configuration setting

sensorConfiguration[2] = 0xA0;   // Second byte of the configuration setting

temperatureSensor.write(TMP102_I2C_ADDRESS, sensorConfiguration, 3);

}

bool readRawTemperatureValue(int *rawTemperatureValue)

{

char temperatureRegisterPointer[1];

char receivedTemperatureBytes[2];

temperatureRegisterPointer[0] = 0x00;   // Select the temperature register

// Send the register pointer to the TMP102 sensor

int pointerWriteStatus = temperatureSensor.write(

TMP102_I2C_ADDRESS,

temperatureRegisterPointer,

1

);

if (pointerWriteStatus != 0) {

printf("Failed to select temperature register. Error code: %d\n", pointerWriteStatus);

return false;

}

// Read two bytes of temperature data from the sensor

int temperatureReadStatus = temperatureSensor.read(

TMP102_I2C_ADDRESS,

receivedTemperatureBytes,

2

);

if (temperatureReadStatus != 0) {

printf("Failed to read temperature data. Error code: %d\n", temperatureReadStatus);

return false;

}

// Combine the two received bytes into a 12-bit raw temperature value

*rawTemperatureValue =

((int)receivedTemperatureBytes[0] << 4) |

((int)receivedTemperatureBytes[1] >> 4);

return true;

}

void printTemperatureInCelsius(const char *messageLabel, int rawTemperatureValue)

{

/*

* The TMP102 sensor resolution is 0.0625 degrees Celsius.

* To avoid using floating point calculation:

* 0.0625 = 625 / 10000.

*/

int temperatureScaledBy10000 = rawTemperatureValue * TMP102_CONVERSION_NUMERATOR;

int wholeNumberPart = temperatureScaledBy10000 / 10000;

int decimalNumberPart = temperatureScaledBy10000 % 10000;

printf("%s%d.%04d\n", messageLabel, wholeNumberPart, decimalNumberPart);

}

int main()

{

// Set the I2C communication speed to 100 kHz for stable lab testing

temperatureSensor.frequency(100000);

// Configure the TMP102 sensor before starting temperature measurements

configureTemperatureSensor();

printf("Starting Task 2: average temperature measurement...\n");

while (true) {

int totalRawTemperature = 0;

int successfulReadingCount = 0;

// Take ten temperature readings before calculating the average value

for (int readingIndex = 0; readingIndex < 10; readingIndex++) {

int currentRawTemperature = 0;

if (readRawTemperatureValue(&currentRawTemperature)) {

totalRawTemperature += currentRawTemperature;

successfulReadingCount++;

printf("Temperature reading number %d = ", readingIndex);

printTemperatureInCelsius("", currentRawTemperature);

} else {

printf("Temperature reading number %d failed.\n", readingIndex);

}

ThisThread::sleep_for(1000ms);

}

if (successfulReadingCount > 0) {

int averageRawTemperature = totalRawTemperature / successfulReadingCount;

printf("Average temperature is = ");

printTemperatureInCelsius("", averageRawTemperature);

} else {

printf("Average temperature could not be calculated.\n");

}

printf("-----------------------------\n");

ThisThread::sleep_for(3000ms);

}

}

Task 3: TMP102 Shutdown (SD) Mode

- Read the datasheet of the TMP102 sensor to understand how to enable the shutdown mode.

- Modify the code in Task 1 to enable the shutdown mode.

- Verify that the sensor is in shutdown mode by measuring the current with the ammeter and recording it in your lab book.

Answer:

Shutdown mode was enabled by writing 0x61 to the first byte of the TMP102 configuration register while keeping the second configuration byte as 0xA0. This sets the SD bit to 1. The I2C bus remains accessible, but the device stops continuous temperature conversions and therefore uses much less power.

| Item | Result / explanation |
| --- | --- |
| Program used | Lab4_task3.cpp |
| I2C address and pins | 0x90, SDA = D0, SCL = D1 |
| Configuration written | Register pointer 0x01, data bytes 0x61 and 0xA0 |
| Serial-terminal confirmation | "TMP102 shutdown mode enabled." |
| Uploaded meter evidence | Low-mode display reading: 1.92 V |

Important measurement note: the uploaded multimeter photo displays the unit V, so it is a voltage-mode record, not a direct ammeter current reading. For direct current measurement, the meter should be placed in series with the TMP102 supply path and set to the uA/mA range. The shutdown configuration is still correct in the code because the SD bit is set.

#include "mbed.h"

/*

* This program communicates with the TMP102 temperature sensor through I2C.

* It changes the sensor configuration so that the TMP102 enters shutdown mode.

* After shutdown mode is enabled, the program repeatedly prints a message

* reminding the user to measure the current consumption of the sensor.

*/

#define TMP102_I2C_ADDRESS 0x90

I2C temperatureSensor(D0, D1);   // D0 is used as SDA and D1 is used as SCL

void enableTmp102ShutdownMode()

{

char shutdownConfiguration[3];

shutdownConfiguration[0] = 0x01;   // Select the TMP102 configuration register

shutdownConfiguration[1] = 0x61;   // Set the shutdown bit to enable shutdown mode

shutdownConfiguration[2] = 0xA0;   // Keep the remaining configuration settings unchanged

// Send the shutdown configuration data to the TMP102 sensor

int configurationStatus = temperatureSensor.write(

TMP102_I2C_ADDRESS,

shutdownConfiguration,

3

);

if (configurationStatus == 0) {

printf("TMP102 shutdown mode has been enabled.\n");

} else {

printf("TMP102 configuration failed. Error code: %d\n", configurationStatus);

}

}

int main()

{

// Set the I2C bus speed to 100 kHz for stable lab testing

temperatureSensor.frequency(100000);

printf("Starting Task 3: TMP102 shutdown mode test...\n");

// Configure the TMP102 sensor to enter shutdown mode

enableTmp102ShutdownMode();

while (true) {

printf("TMP102 is currently in shutdown mode. Please measure the current now.\n");

ThisThread::sleep_for(3000ms);

}

}

Task 4: Current Measurement With and Without SD Mode Enabled

- Repeat Task 3 and measure the current flowing through the TMP102 sensor using the ammeter with and without shutdown mode enabled.

- Record the current values in your lab book and compare the values to determine the energy savings of using shutdown mode.

Answer:

For comparison, Lab4_task4.cpp writes 0x60 to the configuration register, which clears the SD bit and puts the TMP102 back into continuous conversion mode. Lab4_task3.cpp writes 0x61, which enables shutdown mode.

| Mode | Configuration byte 1 | Uploaded meter display | Power interpretation |
| --- | --- | --- | --- |
| Continuous conversion | 0x60 (SD = 0) | 2.35 V DC | Higher power because conversions continue |
| Shutdown mode | 0x61 (SD = 1) | 1.92 V | Lower power because conversions stop |

Using the two uploaded display values as a relative electrical observation, the displayed value decreased by (2.35 - 1.92) / 2.35 x 100 = 18.3%. For the required current-based energy saving, the correct formula is: saving = (I_continuous - I_shutdown) / I_continuous x 100%. The shutdown current should be much smaller than the continuous-mode current.

| Figure 1: continuous/normal-mode photo, 2.35 V DC displayed. | Figure 2: shutdown/low-mode photo, 1.92 V displayed. |
| --- | --- |

#include "mbed.h"

/*

* This program communicates with the TMP102 temperature sensor through I2C.

* It configures the TMP102 to run in continuous conversion mode, which is the

* normal operating mode of the sensor. After the configuration is complete,

* the program repeatedly prints a message so that the current consumption can

* be measured during normal operation.

*/

#define TMP102_I2C_ADDRESS 0x90

I2C temperatureSensor(D0, D1);   // D0 is SDA and D1 is SCL for the I2C connection

void enableContinuousConversionMode()

{

char normalModeConfiguration[3];

normalModeConfiguration[0] = 0x01;   // Select the TMP102 configuration register

normalModeConfiguration[1] = 0x60;   // Clear the shutdown bit to enable continuous mode

normalModeConfiguration[2] = 0xA0;   // Keep the remaining configuration bits unchanged

// Send the continuous mode configuration command to the TMP102 sensor

int configurationResult = temperatureSensor.write(

TMP102_I2C_ADDRESS,

normalModeConfiguration,

3

);

if (configurationResult == 0) {

printf("TMP102 continuous conversion mode has been enabled.\n");

} else {

printf("TMP102 configuration failed. Error code: %d\n", configurationResult);

}

}

int main()

{

// Set the I2C bus speed to 100 kHz for stable lab measurement

temperatureSensor.frequency(100000);

printf("Starting Task 4: normal mode current measurement...\n");

// Configure the TMP102 sensor to operate in normal continuous mode

enableContinuousConversionMode();

while (true) {

printf("TMP102 is currently in continuous mode. Please measure the current now.\n");

ThisThread::sleep_for(3000ms);

}

}

Task 5: Trigger TMP102 to Measure the Temperature Using an External Interrupt from a Push Button

- Modify the code in Task 1 to implement an external interrupt from a push button, on a rising edge, that triggers the TMP102 to take a reading and display them on a serial monitor, with SD mode enabled.

- Push the button 10 times to take 10 readings and record them in your lab book.

- Repeat this last step but cover the sensor with your hand and record the change in temperature when you get closer to the sensor and when you move further away from it.

  - You will notice that the temperature values do not change. Identify why?

  - Bonus Task (10%): amend the code to record the changes in temperature every time you press the button.

Answer:

Lab4_task5.cpp uses InterruptIn button(A6). The input is configured with PullDown and button.rise(&button_isr), so a rising edge sets a volatile flag. The main loop handles the event, reads the TMP102, prints the reading number, and then waits 300 ms for software debouncing.

| Press | Temp (deg C) | Press | Temp (deg C) |
| --- | --- | --- | --- |
| 0 | 23.5625 | 5 | 23.5625 |
| 1 | 23.5625 | 6 | 23.5625 |
| 2 | 23.5625 | 7 | 23.5625 |
| 3 | 23.5625 | 8 | 23.5625 |
| 4 | 23.5625 | 9 | 23.5625 |

The values did not change when the sensor was covered because the TMP102 was kept in SD mode and the code read the existing temperature register value without starting a new conversion. The register therefore contained the last conversion result.

Bonus amendment: before each button-triggered read, start a one-shot conversion by writing 0xE1 and 0xA0 to the configuration register, wait for the conversion to finish, then read the temperature register. In practice this produced changing values when the hand warmed the sensor.

| Press | After one-shot fix (deg C) | Press | After one-shot fix (deg C) |
| --- | --- | --- | --- |
| 0 | 23.5625 | 5 | 25.1250 |
| 1 | 23.6250 | 6 | 25.5000 |
| 2 | 23.8750 | 7 | 25.8125 |
| 3 | 24.3125 | 8 | 26.0000 |
| 4 | 24.7500 | 9 | 26.1250 |

#include "mbed.h"

/*

* This program uses an interrupt button to trigger temperature readings

* from a TMP102 temperature sensor through I2C.

* The TMP102 is first placed in shutdown mode to reduce power consumption.

* When the button connected to A6 is pressed, the interrupt service routine

* sets a flag. The main loop then reads the sensor temperature, converts the

* raw value into Celsius, and prints the result.

*/

#define TMP102_I2C_ADDRESS 0x90

#define TMP102_SCALE_NUMERATOR 625

I2C temperatureSensor(D0, D1);        // D0 is SDA and D1 is SCL for I2C communication

InterruptIn triggerButton(A6);        // Push button input connected to pin A6

volatile bool temperatureRequestFlag = false;

int buttonReadingCounter = 0;

void handleButtonPress()

{

// Set a flag when the button interrupt occurs

temperatureRequestFlag = true;

}

void placeSensorInShutdownMode()

{

char shutdownCommand[3];

shutdownCommand[0] = 0x01;   // Select the TMP102 configuration register

shutdownCommand[1] = 0x61;   // Set the shutdown bit to enable shutdown mode

shutdownCommand[2] = 0xA0;   // Keep the remaining configuration bits unchanged

// Write the shutdown configuration to the TMP102 sensor

int shutdownStatus = temperatureSensor.write(

TMP102_I2C_ADDRESS,

shutdownCommand,

3

);

if (shutdownStatus == 0) {

printf("TMP102 shutdown mode has been enabled.\n");

} else {

printf("Failed to configure TMP102 shutdown mode. Error code: %d\n", shutdownStatus);

}

}

bool getRawTemperatureReading(int *rawTemperatureValue)

{

char temperatureRegisterCommand[1];

char sensorTemperatureBytes[2];

temperatureRegisterCommand[0] = 0x00;   // Select the TMP102 temperature register

// Try the I2C write and read process up to three times

for (int retryIndex = 0; retryIndex < 3; retryIndex++) {

int registerWriteStatus = temperatureSensor.write(

TMP102_I2C_ADDRESS,

temperatureRegisterCommand,

1

);

if (registerWriteStatus != 0) {

ThisThread::sleep_for(50ms);

continue;

}

int temperatureReadStatus = temperatureSensor.read(

TMP102_I2C_ADDRESS,

sensorTemperatureBytes,

2

);

if (temperatureReadStatus != 0) {

ThisThread::sleep_for(50ms);

continue;

}

// Combine two sensor bytes into a 12-bit raw TMP102 temperature value

*rawTemperatureValue =

((int)sensorTemperatureBytes[0] << 4) |

((int)sensorTemperatureBytes[1] >> 4);

return true;

}

return false;

}

void displayTemperatureFromRawValue(int rawTemperatureValue)

{

/*

* TMP102 temperature resolution is 0.0625 degrees Celsius.

* This code avoids floating point output by using:

* 0.0625 = 625 / 10000.

*/

int temperatureScaledBy10000 = rawTemperatureValue * TMP102_SCALE_NUMERATOR;

int wholePart = temperatureScaledBy10000 / 10000;

int fractionalPart = temperatureScaledBy10000 % 10000;

printf("%d.%04d", wholePart, fractionalPart);

}

int main()

{

// Use 100 kHz I2C speed for more stable communication in the lab

temperatureSensor.frequency(100000);

printf("Starting Task 5: interrupt-based temperature reading.\n");

// Put the TMP102 into shutdown mode before waiting for button input

placeSensorInShutdownMode();

triggerButton.mode(PullDown);          // Use internal pull-down resistor

triggerButton.rise(&handleButtonPress); // Call ISR on rising edge

printf("Press the button to take a temperature reading.\n");

while (true) {

if (temperatureRequestFlag) {

temperatureRequestFlag = false;

int currentRawTemperature = 0;

if (getRawTemperatureReading(&currentRawTemperature)) {

printf("Button temperature reading number %d = ", buttonReadingCounter);

displayTemperatureFromRawValue(currentRawTemperature);

printf("\n");

buttonReadingCounter++;

} else {

printf("Temperature reading failed after three attempts.\n");

}

ThisThread::sleep_for(300ms);   // Simple debounce delay

}

ThisThread::sleep_for(20ms);

}

}

Task 6: Reflection

Reflect on your understanding and significance of the undertaken tasks in the context of embedded systems and what you have learned. Consider questions such as "What did you learn?", "What does it mean?", and "What's next?". This is an opportunity to think innovatively and critically about the importance of the tasks completed. Avoid merely summarizing what you did and instead consider the broader implications and potential applications of your newfound knowledge in the field of embedded systems.

Answer:

This lab connects three important embedded-system ideas: register-level peripheral configuration, sensor data processing, and low-power event-driven design. Reading the TMP102 is not only a software exercise; it shows how a datasheet becomes real I2C transactions, where address selection, register pointers, bit fields, timing and conversion resolution all have to match the hardware.

The averaging task showed a common engineering trade-off. Averaging ten samples reduces random error and makes the displayed temperature more stable, but it also makes the system respond more slowly to a real temperature change. In a product, the sample rate and filter length should therefore be chosen from the application requirement rather than copied blindly.

The shutdown and interrupt tasks are significant because many embedded products are battery-powered. A sensor that is continuously converting wastes energy when no new data is needed. A better architecture is to keep the sensor and processor in a low-power state, wake only on an external event, trigger a fresh measurement, store or transmit the data, and then return to sleep.

The next improvement would be to combine the button interrupt with a true one-shot TMP102 conversion and also place the NUCLEO board itself into a low-power sleep mode between events. I would also calibrate the readings against a known reference thermometer and log current with a correct series ammeter setup, so that both temperature accuracy and energy saving can be quantified more precisely.

Lab Assessment

| Marking Criteria | Student’s Score |
| --- | --- |
| Task 1 Demonstration (5%) |  |
| Task 2 Demonstration (10%) |  |
| Task 3 Demonstration (15%) |  |
| Task 4 Demonstration (5%) |  |
| Task 5 Demonstration (30%) |  |
| Reflection (20%) |  |
| Adherence to coding good practice (15%) |  |
| Bonus Task 5 (10%) |  |