# Lab 1 – Specification-Driven Embedded System Design

Embedded Processors UESTC2004 & UESTCHN2004 School of Engineering, University of Glasgow

| Student Name: |  |
| --- | --- |
| Student Number: |  |

### Lab Assessment Criteria (UESTC 2004)

Each lab will be evaluated based on the following criteria:

### Demonstrating a Working Embedded System and Answering Lab Questions (70%)

- To earn full marks, you must show your system working correctly for each task in the lab manual.

- You will need to demonstrate this to the Graduate Teaching Assistant (GTA) and a staff member.

### Reflection and Understanding (5%)

After completing each task, write a short reflection to explain:

- What you did – Describe the task you worked on.

- What it means – Explain its purpose or significance in the context of embedded systems.

- What’s next – Think about how this task connects to what you're learning in class or how it might apply in real-world scenarios.

💡 Be creative and think critically — avoid simply summarising what you did, as summaries won't be accepted.

### Following Good Coding Practices (25%)

To score well in this area, make sure you:

- Use meaningful variable names – Avoid vague names like x, y, or personal names.

- Add comments to your code – Use block comments at the start to describe your code’s overall purpose and line comments to explain specific lines of code.

- Follow consistent indentation – Proper indentation makes your code easier to read and understand.

Here’s an example of best practices to follow and mistakes to avoid:

Good practice 🗸

#include “mbed.h”

char switch_word ;		//the word we will send char recd_val;	//the received value

int main() {

async_port.baud(9600);	//set baud rate to 9600    recd_val = 0;

while (1)

{

switch_word=0xa0;

}

}

Avoid this ✘

#include “mbed.h”

char switch_word ;

char recd_val;

int main() {

async_port.baud(9600);

recd_val = 0;

while (1)

{

switch_word=0xa0;

}

}

### Lab Objective

The purpose of this laboratory is to introduce specification-driven embedded system design. Instead of following detailed programming instructions, you will interpret a set of functional and technical requirements and implement an embedded system that satisfies them.

By the end of this laboratory, you should be able to:

- translate system specifications into embedded software behaviour

- structure embedded software using modular functions

- implement timing behaviour using software techniques

- generate and verify digital signals using an oscilloscope

- use analogue input to control system behaviour

- validate embedded system performance against requirements

This lab focuses on system design rather than step-by-step instructions.

System Overview:

In this laboratory exercise, you will design and implement a Programmable Signal Controller (PSC). Your goal is to translate a written functional specification into a working implementation, demonstrating how embedded systems evolve from requirements into real behaviour.

The PSC operates in three distinct modes, each defining how the system responds to user input and how it generates output signals:

Mode 0 – Idle:

In this resting state, the controller waits for user interaction. No output signal is generated, but the system remains responsive to mode changes.

Mode 1 – Square Wave Generator:

In this mode, the device produces a square wave digital signal. The fundamental frequency of this signal is controlled via an analogue input, allowing the user to adjust it dynamically using a potentiometer.

Mode 2 – Pattern Generator:

Here, instead of producing a simple square wave signal, the system outputs a predefined or programmable digital pattern. This illustrates how embedded systems can perform more complex timed behaviours.

To support these features, the PSC makes use of several hardware inputs and outputs:

- 1 LED indicator – used to convey system status.

- 2 push buttons – one for cycling through modes, one for resetting the system.

- 1 analogue input (potentiometer) – used to adjust parameters such as output frequency.

- 1 digital signal output – used to generate either the square wave signal or the digital pattern, depending on the mode.

For convenience, the following pin assignments are suggested:

- LED → A6

- Signal Output → D5

- Mode Button → D10

- Reset Button → D11

- Potentiometer → A0

Throughout this lab, you will interpret the system specification, design appropriate software architecture, and implement each mode’s behaviour. By the end, you will have built a flexible embedded controller driven entirely by the requirements, illustrating the core principles of specification‑driven development.

System Behaviour Overview:

The PSC system is expected to run continuously and relies solely on software‑based timing, without hardware timers, interrupts, or RTOS utilities. Short, non‑blocking delays (less than 5ms) must be used to achieve the timing requirements, and overall behaviour is expected to remain within ±10% of the stated values.

The controller begins in Mode 0 (Idle Mode). In this state, the LED remains off, and the signal output is held low. The system performs no active signal generation until the user presses the mode button, at which point it transitions to the next mode.

In Mode 1 (Square Wave Generator), the LED provides visible feedback by flashing at approximately 1 Hz, indicating that the system is actively generating a periodic waveform. The output pin produces a square wave whose period is controlled by the analogue input. The potentiometer sets the fundamental frequency smoothly across a defined range, from 100 Hz at its minimum position to 2000 Hz at its maximum, allowing the user to vary the output continuously.

In Mode 2 (Pattern Generator), the LED switches to a permanently on state, signalling that the controller is now outputting a timed digital pattern. The signal output follows a repeating sequence:

- HIGH for 10 ms

- LOW for 20 ms

- HIGH for 40 ms

- LOW for 80 ms

This four-step pattern repeats continuously for as long as the system remains in Mode 2.

Two push buttons control the system. The Mode button cycles through the three operating modes sequentially, i.e., Idle → Frequency Generator → Pattern Generator → Idle → …, with each press advancing to the next state. At any time, the Reset button immediately returns the system to Mode 0 (Idle).

This behaviour specification forms the foundation for your implementation task. Your challenge is to translate these requirements into reliable embedded software that respects the timing constraints and mode logic while operating entirely through software-controlled delays.

Task 1: Software Architecture

Before writing code, design the program structure. Your program should clearly separate:

- input reading

- mode control

- signal generation

- LED control

Draw a diagram showing the software architecture and the transition from one state to the next in your lab notebook.

[Provide answers and evidence below, this could be photos taken during the lab]

### Task 2: PSC Mode Control Implementation

Implement the PSC Mode Control using LEDs only. Expected behaviour:

- Mode 0 → LED OFF

- Mode 1 → LED blinking

- Mode 2 → LED ON

[Provide answers and evidence below, this could be photos taken during the lab]

Task 3: PSC Square Wave Generator

Implement the frequency generator for Mode 1 and integrate it with Task 2. Requirements:

- Square wave output

- Frequency range 100 Hz – 2000 Hz

- Frequency controlled by potentiometer

Measure the signal using an oscilloscope. Record the following measurements:

| Potentiometer Position | Potentiometer Value | Expected Frequency | Measured Frequency |
| --- | --- | --- | --- |
| Minimum |  |  |  |
| Middle |  |  |  |
| Maximum |  |  |  |

Calculate the approximate percentage error between expected and measured values.

[Provide answers and evidence below, this could be photos taken during the lab]

Task 4: PSC Pattern Generator

Implement the Mode 2 waveform and integrate it with Task 3. Required pattern:

- HIGH → 10 ms

- LOW → 20 ms

- HIGH → 40 ms

- LOW → 80 ms

Measure the cycle using an oscilloscope.

[Provide answers and evidence below, this could be photos taken during the lab]

## Reflection

After the completion of all the tasks please reflect, in a few lines, on your understanding of the tasks and its significance. This is a chance for you to ask yourselves “What you did?”, “What it means?”, “What next?”, and/or “How else, more efficiently, can this be done?”, meaning what is the significance of the undertaken task in the context of embedded systems and what you are learning during lectures. Try to be innovative and think outside the box. Please, do not provide a summary of what you did as it will not be accepted.

[Answer in the space below]

## Lab Score

| ## Marking Criteria | ## Student’s Score | ## Comments from the GTA |
| --- | --- | --- |
| ## Demonstration Task 1 (10%) |  |  |
| ## Demonstration Task 2 (20%) |  |  |
| ## Demonstration Task 3 (25%) |  |  |
| ## Demonstration Task 4 (20%) |  |  |
| ## Reflection (5%) |  |  |
| ## Adherence to coding good practice (20%) |  |  |